p084r: Eau magistra

Fais dissouldre le sel gemme ou sel suin de verre subtilem{ent}

pulverise & mis sur un mabre dans une cave & il se desseichera

en recuisant le moule & luy donnera une liaison pour soubstenir

plusieurs gects Essaye de humecter avecq huile de tartre

p084r: Eau Magistra

Dissolve rock salt or sandever that you have pulverized finely and placed on a marble slab in a pot. It will dry out while you reheat the mold, and will provide it with a binding to enable several castings. Try moistening it with tartar oil.

The entry in p084r for “eau magistra,” which translates literally to water magistry, refers to a liquid mixture that is added into casting sands so that the sand will hold together. Variations of an eau magistra in our manuscript call for ingredients such as elm root and wine (or vinegar), and burnt oysters. The eau magistra in p084r calls for finely ground and dissolved sandever or rock salt, moistened in tartar oil.

Magistry recipes, even if identical to “binders” in function, are strangely made distinct from other “binder” materials. For example, in p085v the author practitioner suggests that egg whites be beaten together with earth to make a casting sand; the egg whites should help make the impression “come clean and sets and stabilizes the material;” however, egg whites are not described as a magistry. Though the distinction between a magistry and a binder is not further elaborated in our manuscript, a few other contemporaneous sources define magistry as a liquid concoction that contains a wine or vinegar combined with a “salt.” For example, in a recipe detailing the use of the “salt of [oriental] pearls,” the author writes in a footnote that “it is call'd Salt, because it resolves like Salt, but properly it is a Magistery. For they are called Magisteries, in which the whole remains intire…freed from their grosseness by the sharp Spirits (wine and vinegar), [the salts of pearl] are visibly seen to come forth.” Along with pearl, coral was also used as a “salt” for magistry recipes.

Nonetheless, the magistry recipes and other binders in the manuscript generally share three main objectives; to help the sand bind well together, create a clean impression, and last more than one casting. These three objectives also illustrate a fascinating connection between the sand and binder. A casting sand is often described as both dry and lean, and requires something fatty or moist to bind it together. For instance, in describing sand made from the hoof of an ox in p084v, the author practitioner writes that “on its own it makes a clean mold. But because on its own it is very dry and lean, it demands to be well wet and humidified, with a thick broth with elm root.” The author practitioner writes in p085r that “then I knew that sands used to mold big relief must be very moist with some kind of water, which gives body and firmness, like egg white, gummed water, [or] one [water] boiled with elm root….It can take as many firings as you want because it is as hard as glass.” In addition, he writes on the same manuscript page that a “fat metal needs a lean sand,” using the same terminology to define the optimal combinations of metals and casting sands. It is apparent that knowing what dry, moist, lean and fatty mean is integral to understanding the properties of casting sands. Yet we wondered how a material is defined by these properties, and made it one of our main objectives to answer this inquiry.

Though it may seem at first glance that the terms dry and moist used in sand casting recipes refer to the four humors, a closer look shows that both dry and wet/moist are mainly used to support the terms lean and fatty, which are not a part of the humoral system. In fact, fatty and lean are very distinct from the humors and, according to Pamela Smith, were originally used to describe the qualities of soil in the study of agriculture, most notably in the works of Agricola. The fat/lean dichotomy runs abundant in the casting recipes in our manuscript, and is especially pertinent to our other experiment on sand made from calcinated ox bone hooves and rock salt in p089r. Yet we struggled with the concept of fat and lean; reading these descriptions off of a page, without actually knowing the material in that manner, proved to be initially useless.

Our first actual experience with fat and lean occurred when we ground pink Himalayan rock salt for both our magistry and ox bone/rock salt experiments. After grinding the salt in rock mortars as finely as possible and storing it back into an air-tight jar, we returned to the lab a few days later and found that the texture of the salt was very “sticky.” The sensation is incredibly difficult to describe, other than that the salt was not “dry” like flour, which would pour smoothly from an outstretched hand; the salt particles were clearly sticking to each other, and subsequently created a very odd sensation of resistance when running fingers along the grains. Rediscovering a material that we eat and use every day was rather uncanny, but it helped us to reevaluate a very common material through a highly sensory and material sort of context. Instead of simply knowing that salt is chemically known as sodium chloride, we felt the “fattiness” of the rock salt and reintroduced ourselves to it as a workable material in craft.

[Figure 1]

As a result, what we discovered with fat and lean is that these terms rely very heavily upon the senses and direct interaction with materials. In order to work with any material in the early modern world, it was absolutely essential that an ingredient was fully understood through all five senses of the body; for example, Agricola writes that a very unctuous soil will melt like butter on the tongue, implying the use of taste. In fact, the body itself was a tool in experimentation; not only was the body used to study a material, but “the human body was also a source of substances employed in manufacture…and provided a conceptual framework for the transformation of materials in nature…the workshop functioned as an extension of the capacities and products of the human body.”

Just like how in modern science we define and understand materials through their chemical makeup, early modern practitioners knew them intimately and through their bodies. This direct relationship between practitioner and material was so fundamentally intertwined that it often was connected to – and cost them – their health; Smith writes that “the body…was also implicated in the work: the bodies of metalworkers and the very matter upon which they labored interpenetrated each other: bad breath could prevent the adhesion of metal gilding, and, conversely, metal fumes were known to shorten the lives of metalworkers.” This “interpenetration” between practitioner and material indicates that early modern experimentation was so closely related to the human body that the effects of either the practitioner or his material could deeply affect the outcome of the work.

In fact, the connection is so strong that the four humors and temperaments played a huge role in maintaining the balance of health and the materials in the work place. Bad breath, bread, and even butter were materials and factors that “operated within the Greco-roman-arabic health worldview based on an Aristotelian framework of the four elements and qualities…this was also the framework for the explanation from mine manager, engineer and author of De la Pirotechnia (1540) Vannoccio Biringuccio for what happened when bronze was poured into a cold mould – the cold, moist mould overcame the nature of bronze and turned it back to its primordial earthy-watery state.” By following the framework of the four humors, the practitioner ensured success by making sure that all aspects of his experimentation were balanced and interacted with each other by corresponding to their humoral relationships.

The sheer importance of sensory interactions with the material resulted in one of the biggest frustrations with our magistry experiment, because we were unable to obtain sandever. What we do know of it also ascribes to the fatty/lean framework. For example, sandever is defined as the “fatty substance floating on glass when it is red-hot in the furnace, and which being cold is as hard as stone, yet brittle, and easily broken” in a French-English dictionary published in 1611. When taken out from the glass and cooled down to a solid, the glass gall “forms a white crumbly mass, sometimes quite white and at other times brown and fouled, and strongly saline, but not very uniform in its composition: being sometimes merely salt, often very bitter, probably as common salt or sulphate of potash predominates.” Since the material of the glass could and most likely did vary, the actual chemical composition of sandever seems to be heavily dependent on the glass’s composition, which may mean that no two batches of glass gall would have the same chemical make up; however, sandever seems to have been consistently bitter and salty. In terms of actual composition, sandever is comprised of “all those salts contained in common alkalies which readily melt at somewhat less than a glass-making heat, and are either naturally considerably volatile, or have little if any affinity for silex, and do not unite in the composition of glass, but, being superficially lighter, rise to the top.”

It is also interesting to note that glass gall is “very volatile in a strong fire…it is generally scummed off with iron ladles, and sold to metal refiners as a powerful flux.” This strongly suggests that glass gall was easily and readily made in glass-making shops of a significant enough quality to be sold and traded as a commercial product. As a flux in metal-working, the sandever presumably would have been used to purify metals for both metal joining and extractive metallurgy.

However, sandever’s usages are not limited to metal working, but were also used as an ingredient for Crocus Metallorum in addition with antimony and salt-peter. In addition, sandever was used as a pesticide for garden walks, where it “destroys both weeds and vermin,” and was even sold in France “to powder their meat, and to eat, instead of common salt.” However, it seems that though there was at least a moderate industry for sandever, it was not a material fully understood even in the early modern period. De Blancourt writes that “this salt which the French call (Suin de Verre) Sandever is useful for several purposes, and in several Chymical Operations. It has besides some other Uses and Virtues, that are not known, even to very few of the Learned; I could tell some very surprising and wonderful uses of it. But this may serve to whet the Industry of, and excite the Curious to further Enquiries.”

Unfortunately, because modern glass making technologies and strict quality control of glass material ensure that byproducts are not produced in glass production, sandever is now an obsolete material that is almost impossible to obtain. Though there are refineries that create usable glass from waste products, the sandever produced from these refining processes are too toxic to use for historical recreation because they contain asbestos. With no feasible way to obtain sandever for the magistry experiment, we decided to follow the recipe’s suggestion and use rock salt in lieu of sandever.

We were also eager to explore why the author practitioner suggests that either rock salt or sandever can be used for the magistry recipe. We hypothesized that the interchangeability of rock salt and sandever strongly implies that they are materials that are not only very closely related to each other, but also perform identically as casting materials. For example, the author of a recipe from the eighteenth century suggested that “pig or sow iron, videlicet, the ashes of wood and other vegetables, all kinds of glass and sandever, common salt and rock salt” be mixed into the iron fusion to purify the iron, demonstrating the same idea of interchangeability. Biringuccio also categorized rock salt in a family of salts that include potash and sal ammoniac.

However, one of the most telling pieces of evidence we found to support our conjecture was the fact that both sandever and rock salt are suggested for use as dry binding materials in different recipes; surprisingly, both materials react identically with their lean components. For example, the recipe directly following our magistry recipe on p084r titled “Sand from a Toulousain Mine” instructs the reader to use sandever as an ingredient in the sand itself. The sand, mined deep from the earth, “is excellent on its own, but to make it sustain multiple castings, I mix it with pulverized and moistened sandever, which hardens it, holds well together with it, and enables as many castings as you like.” Coincidentally, our experiment with ox bone and rock salt in p089r similarly suggests that the reader pulverize and moisten the ox bone and rock salt; the combined ox bone and rock salt molds hardened to a cement-like firmness, held well together, and remained intact enough to be cast more than once (please refer to our annotation on ox bone and rock salt, p089r, for more details). Though we could not personally verify the results of the sandever for the Toulousain sand recipe, the end results of both recipes are almost exactly the same, demonstrating that rock salt and sandever most likely behave identically as fatty sands in casting.

Additionally, the two recipes above do not rely upon a wet binder like elm root infusions and egg whites, but rather on the “fatty” properties of the rock salt and sandever to bind the sand together. The fact that sandever and salt are dry materials that successfully work as binders imply that what is most important for the binding effect and the overall balance of a casting sand is dependent on the proper equation of fatty and lean materials, and not upon the humoral dry and wet. Additionally, the transfer of the fatty/lean dichotomy from agriculture to metalworking works perfectly with casting sands because many sands contain fresh soil and dirt; using earth-like materials to cast metal works parallel on a smaller scale the direct relationship between the earth and metals (most importantly manifested in mining). As a result, a metalworker was most concerned with whether or not the casting sand has the perfect amount of fatness and leanness to hold together in the face of hot, molten metals.

Additionally, our magistra recipe may also be unique from the other “eau magistras” in the manuscript, in that it is merely “moistened” with tartar oil and not made “wet.” What the language could imply is that the salt/sandever magistra actually relies largely on the fatty qualities of the sandever and salt to do the binding with the sand. However, this conjecture must be better understood and verified through further research and inquiry.

Our other main source of investigation was to determine whether or not the magistra held the sand well together after the tin casting and retained enough detail, since the recipe specified that the resulting mold should be strong enough to withstand several castings. The short answer to this inquiry is that our double-sided mold remained intact enough for a second clean casting. However, the main issue with our experiment was that we could not definitively attribute the success of our experiment to the binder, because there were too many variables in our experiment that were not controlled and resulted in an unusual outcome with the medal. First, the dry sand we used was reground molds from our previous experiments with metalwork, which is a combination of brick and plaster. In addition, the sand had several binders already mixed into it, such as brandy and sal ammoniac, and some unknown variable had caused a yellow-goldish tinge to cover the surface of our medal. Though at first we believed it to be from the salt, we did not find any of the same effects on our medals made from our ox bone and rock salt molds, even though that sand had a much higher ratio of salt than our binder solution. Other medals previously cast in the same brick/plaster sand had also displayed the same sort of discoloration, though to a lesser extent, but currently it is hard to positively identify what had caused the strange tint.

[Figure 2]

[Figure 3]

[Figure 4]

[Figure 5]

The concluding thoughts on our binder experimentation is that, despite unexpected and uncontrolled variables affecting the cast of the medal itself, our magistra successfully remained intact and detailed enough for a potential second casting. In addition, while interpreting the recipes in our experimentation, we found that the five senses and the relationship between the body and the materials were central to how experimentation was done and understood. Finally, the transfer of the fatty and lean dichotomy from agriculture to metal casting demonstrates the optimal qualities that contributed to the most effective casting sands. Rather than the humoral dry and wet relationship, fatty and lean allowed the metal caster to understand the sand as an earth that could be tailored to interact optimally with whichever metal it was designed for.

Bibliography:

Crollius, Oswald (ca. 1560-1609). Bazilica chymica, & Praxis chymiatricae, or, Royal and practical chymistry in three treatises : wherein all those excellent medicines and chymical preparations are fully discovered, from whence all our modern chymists have drawn their choicest remedies. Translated by John Hartman. 1670.

Smith, Pamela. Anderson, Christy, Dunlop, Anne, and Smith, Pamela. “The matter of ideas in the working of metals in early modern Europe,” in The Matter of Art: Materials, Practices, Cultural Logics, c. 1250-1750, edited by Christy Anderson, Anne Dunlop, and Pamela Smith, 42-68. Manchester: Manchester University Press, 2014.

Cotgrave, Randle. Dictionarie of the French and English Tongues. London: Printed by Adam Islip Anno, 1611.

Unknown, “Sandiver, or Glass Gall.” Glass, Brick, Pottery and Glass Journal 6 (1879): 77

Neri, Antonio (d. 1614). The art of glass, wherein are shown the wayes to make and colour glass, pastes, enamels, lakes, and other curiosities. Translated by Christopher Merret. London: Printed by A.W. for O. Pulleyn, 1662.

de Blancourt, Jean Haudicquer (ca. 1650). The art of glass, showing how to make all sorts of glass, crystal, & enamel ... Illustrated with proper sculptures. London: Printed for D. Brown. T. Bennett, [etc., etc.], 1699.

Payne, Josh. “Decarburization and Purification by Means of Cinder, Ashes, Salt, Silex, Potash and Clay”, Van Nostrand’s Engineering Magazine 1 (1728): 194-195

Biringuccio, Vannoccio. The Pirotechnia of Vannoccio Biringuccio. Translated by Cyril Stanley and Martha Teach Gnudi. New York: Dover Publications, 1990.

“Eau Magistra” in p087v of the manuscript

“Eau Magistra” in p084v of the manuscript

Oswald Crollius (ca. 1560-1609), Bazilica chymica, & Praxis chymiatricae, or, Royal and practical chymistry in three treatises : wherein all those excellent medicines and chymical preparations are fully discovered, from whence all our modern chymists have drawn their choicest remedies. trans. John Hartman. (1670), 73

Crollius, 44

Pamela Smith, “The matter of ideas in the working of metals in early modern Europe,” in The Matter of Art: Materials, Practices, Cultural Logics, c. 1250-1750, ed. Pamela Smith et al. (Manchester: Manchester University Press, 2014), 64

Smith, 59

Smith, 44

Ibid

Ibid

Randle Cotgrave. Dictionarie of the French and English Tongues. (London: Printed by Adam Islip Anno, 1611)

Ibid

Unknown, “Sandiver or Glass-Gall” in Brick, Pottery and Glass Journal 6 (1879): 77

Ibid

Antonio Neri (d. 1614), The art of glass, wherein are shown the wayes to make and colour glass, pastes, enamels, lakes, and other curiosities, trans. Christopher Merret (London: Printed by A.W. for O. Pulleyn, 1662), 277

Ibid

Jean Haudicquer de Blancourt (ca. 1650), The art of glass, showing how to make all sorts of glass, crystal, & enamel ... Illustrated with proper sculptures (London: Printed for D. Brown. T. Bennett, [etc., etc.], 1699), 58

Glen Cook (Senior Research Associate at the Corning Glass Museum), email message to D. Mellon titled “RE: query: glass gall,” 11 November 2014.

Ibid

Josh Payne. “Decarburization and Purification by Means of Cinder, Ashes, Salt, Silex, Potash and Clay”, Van Nostrand’s Engineering Magazine 1 (1728): 194.

Vannoccio Biringuccio, The Pirotechnia of Vannoccio Biringuccio, trans. Cyril Stanley and Martha Teach Gnudi (New York: Dover Publications, 1990), 112